# Network Engineering Home Lab

Getting Started Guide

Cisco Catalyst 2960 Switch & Cisco 2911 Router – Packet Tracer Edition

## Introduction

This guide will help you set up and complete a professional-level networking lab using Cisco Packet Tracer simulation software. By the end of this series, you'll have hands-on experience with VLANs, routing, DHCP, NAT/PAT, ACLs, and security hardening—all skills directly applicable to network and security engineering roles.

Each project builds on the previous one, creating a comprehensive understanding of enterprise network configuration and troubleshooting.

## Lab Equipment

- Cisco 2911 Router (available in Packet Tracer device library)

- Cisco Catalyst 2960 Switch (available in Packet Tracer device library)

- Copper Straight-Through cables (use from Packet Tracer connection menu)

- Console cable (use from Packet Tracer connection menu for initial setup)

- 2 PCs from Packet Tracer device library (under End Devices)

Using Packet Tracer: Packet Tracer is Cisco’s free network simulation software. Download it from netacad.com (requires free account). All configurations in this guide work identically to physical equipment, making this an excellent way to build skills before (or instead of) investing in hardware.

Note on Cable Types: In Packet Tracer, always use “Copper Straight-Through” cables for all connections in this lab (router-to-switch, switch-to-PC). The 2960 switch and 2911 router support Auto-MDIX, so straight-through cables work for everything. You’ll find the cable selection in the orange lightning bolt icon at the bottom of the Packet Tracer interface.

## Packet Tracer Setup

### Step 1: Building Your Network Topology

- Add Router: Click the router icon (orange box) at bottom of screen, select 2911, drag onto workspace

- Add Switch: Click the switch icon (square with arrows), select 2960, drag onto workspace

- Add PCs: Click End Devices icon (computer), select PC, add 2 PCs to workspace (label them PC1 and PC2)

- Connect Devices: Click orange lightning bolt icon, select “Copper Straight-Through”. Connect: Router Gi0/0 → Switch Gi0/1, PC1 → Switch Fa0/2, PC2 → Switch Fa0/3

Key Points for Router-Switch Connection:

- Use the Gigabit uplink ports (Gi0/1 or Gi0/2) on the switch for router connections

- Reserve FastEthernet ports (Fa0/x) on the switch for end devices like PCs

- Both router (Gi0/0) and switch (Gi0/1) have Gigabit ports — link will negotiate to 1000 Mbps (full Gigabit speed)

- Verify connection with: show interfaces gi0/1 status

### Step 2: Accessing Device CLI

In Packet Tracer, you access device CLI by clicking on the device:

- Click on the router or switch icon in your topology

- Select the “CLI” tab at the top of the device window

- You’ll see the device prompt (Router> or Switch>) ready for commands

### Step 3: Verify Topology

- Check connection lights: Green triangles on both ends of cables indicate working connections

- Orange/amber triangles mean devices are booting or ports are admin down

- Test CLI access on both router and switch by clicking each device and selecting CLI tab

## Essential IOS Commands to Know

Before starting projects, familiarize yourself with these Cisco IOS modes and commands:

| Mode | Description & Key Commands |
| --- | --- |
| User EXEC Router> | Limited view mode. Type 'enable' to enter privileged mode. |
| Privileged EXEC Router# | Full viewing rights. Key commands: show running-config, show ip interface brief, show vlan brief |
| Global Config Router(config)# | Configuration mode. Enter with 'configure terminal'. Exit with 'exit' or Ctrl+Z. |
| Interface Config Router(config-if)# | Configure specific interface. Enter with 'interface <type> <number>'. |

Tip: Use TAB for command completion and '?' for help at any time.

## Lab Network Design

Your lab will implement this logical network design:

| VLAN | Name | Purpose |
| --- | --- | --- |
| 10 | Users | End-user workstations |
| 20 | Servers | Server infrastructure |
| 99 | Management | Network device management |

| VLAN | Network | Gateway | DHCP Range |
| --- | --- | --- | --- |
| 10 | 192.168.10.0/24 | 192.168.10.1 | .10-.250 |
| 20 | 192.168.20.0/24 | 192.168.20.1 | .10-.250 |
| 99 | 192.168.99.0/24 | 192.168.99.1 | Static only |

Physical Topology:

- PC1 → Switch Port Fa0/2 (VLAN 10)

- PC2 → Switch Port Fa0/3 (VLAN 20)

- Switch Gi0/1 → Router Fa0/0 (802.1Q Trunk)

## Project 1: VLANs, Trunking, and Inter-VLAN Routing

Duration: 2-3 hours

Objective: Create network segmentation using VLANs and enable communication between VLANs using Router-on-a-Stick.

### Configuration Tasks

A. Switch Configuration

Create VLANs:

Switch> enable Switch# configure terminal Switch(config)# vlan 10 Switch(config-vlan)# name Users Switch(config-vlan)# exit Switch(config)# vlan 20 Switch(config-vlan)# name Servers Switch(config-vlan)# exit Switch(config)# vlan 99 Switch(config-vlan)# name Management Switch(config-vlan)# exit

Assign Access Ports:

Switch(config)# interface fastEthernet 0/2 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 10 Switch(config-if)# exit Switch(config)# interface fastEthernet 0/3 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 20 Switch(config-if)# exit

Configure Trunk Port:

Switch(config)# interface gigabitEthernet 0/1 Switch(config-if)# switchport mode trunk Switch(config-if)# switchport trunk allowed vlan 10,20,99 Switch(config-if)# exit Switch(config)# exit Switch# write memory

### Understanding Router-on-a-Stick & Management VLAN

Before configuring the router, let’s understand what we’re building and why.

The Problem: Right now, your VLANs are isolated. A device in VLAN 10 cannot communicate with a device in VLAN 20. VLANs operate at Layer 2 (Data Link), but to communicate between them, you need Layer 3 (Network) routing.

What is Router-on-a-Stick?

Router-on-a-Stick is a configuration where a single physical router interface is divided into multiple virtual subinterfaces, each handling a different VLAN. This allows inter-VLAN routing using only one cable connection.

Traditional Approach (Expensive):

- Router Fa0/0 → Switch VLAN 10

- Router Fa0/1 → Switch VLAN 20

- Router Fa0/2 → Switch VLAN 99

Problem: Requires 3 physical cables and 3 router ports!

Router-on-a-Stick Approach (Efficient):

- Router Gi0/0 (physical) → Switch Gi0/1 (trunk)

- ├─ Gi0/0.10 (subinterface) → VLAN 10

- ├─ Gi0/0.20 (subinterface) → VLAN 20

- └─ Gi0/0.99 (subinterface) → VLAN 99

Solution: Only 1 cable and 1 router port needed!

How Traffic Flows (Example):

When PC1 (VLAN 10) pings PC2 (VLAN 20):

- PC1 sends packet to its gateway (192.168.10.1)

- Switch tags packet with VLAN 10 and sends up trunk (802.1Q tagging)

- Router receives on Gi0/0, reads VLAN 10 tag, routes to Gi0/0.10 subinterface

- Router checks destination (192.168.20.x), routes to Gi0/0.20 subinterface

- Router tags packet with VLAN 20 and sends back down trunk

- Switch reads VLAN 20 tag, forwards to Fa0/2, PC2 receives packet

What About VLAN 99 (Management)?

VLAN 99 is your Management VLAN. It’s used to remotely access and manage your network devices (switches, routers). To make the switch manageable over the network, you need to give it an IP address:

Switch Management Configuration (do this after router config):

Switch(config)# interface vlan 99 Switch(config-if)# ip address 192.168.99.2 255.255.255.0 Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# ip default-gateway 192.168.99.1

This creates an SVI (Switch Virtual Interface) that allows you to SSH or Telnet to the switch from any VLAN once routing is configured.

B. Router Configuration (Router-on-a-Stick)

Router> enable Router# configure terminal Router(config)# interface gigabitEthernet 0/0 Router(config-if)# no shutdown Router(config-if)# exit Router(config)# interface gigabitEthernet 0/0.10 Router(config-subif)# encapsulation dot1Q 10 Router(config-subif)# ip address 192.168.10.1 255.255.255.0 Router(config-subif)# exit Router(config)# interface gigabitEthernet 0/0.20 Router(config-subif)# encapsulation dot1Q 20 Router(config-subif)# ip address 192.168.20.1 255.255.255.0 Router(config-subif)# exit Router(config)# interface gigabitEthernet 0/0.99 Router(config-subif)# encapsulation dot1Q 99 Router(config-subif)# ip address 192.168.99.1 255.255.255.0 Router(config-subif)# exit Router(config)# exit Router# write memory

C. Configure Switch Management IP (VLAN 99)

Now configure the switch with a management IP address so you can access it remotely via SSH or Telnet from any VLAN.

Switch> enable Switch# configure terminal Switch(config)# interface vlan 99 Switch(config-if)# ip address 192.168.99.2 255.255.255.0 Switch(config-if)# description Management Interface Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# ip default-gateway 192.168.99.1 Switch(config)# exit Switch# write memory

What This Does:

- interface vlan 99: Creates a virtual interface (SVI) for the switch itself on VLAN 99

- ip address 192.168.99.2: Gives the switch an IP address so you can ping/SSH/Telnet to it

- ip default-gateway: Allows the switch to reach other networks through the router

Test Management Access:

From the router, you should now be able to ping the switch:

Router# ping 192.168.99.2 (Should succeed!)

Verify VLAN 99 Configuration:

Switch# show vlan id 99 VLAN Name Status Ports ---- -------------------------------- --------- ----------- 99 Management active (Confirms VLAN 99 is active and configured)

Note: SSH and Telnet configuration will come in Project 5 (Switch Security). For now, you have console access and the management IP is configured.

D. Configure PCs with Static IP Addresses

Now configure each PC with a static IP address. Remember, we haven’t set up DHCP yet, so you must manually configure each PC.

PC1 Configuration (VLAN 10 - Users):

- Click on PC1, go to Desktop tab, click IP Configuration

- Select Static (not DHCP)

- Enter the following:

IP Address: 192.168.10.10 Subnet Mask: 255.255.255.0 Default Gateway: 192.168.10.1

PC2 Configuration (VLAN 20 - Servers):

- Click on PC2, go to Desktop tab, click IP Configuration

- Select Static (not DHCP)

- Enter the following:

IP Address: 192.168.20.10 Subnet Mask: 255.255.255.0 Default Gateway: 192.168.20.1

Important: Make sure to select “Static” not “DHCP” since we haven’t configured a DHCP server yet. That comes in Project 2!

### E. Verification Steps

- Verify VLANs: show vlan brief

- Verify Trunk: show interfaces trunk

- Verify Subinterfaces: show ip interface brief

- Test Connectivity: Configure PC1 with IP 192.168.10.10/24, gateway 192.168.10.1

- Configure PC2 with IP 192.168.20.10/24, gateway 192.168.20.1, then ping between them

### F. Test Connectivity from PCs

Now test that inter-VLAN routing is working properly.

From PC1 (Desktop → Command Prompt):

C:\> ipconfig (Verify IP is 192.168.10.10) C:\> ping 192.168.10.1 (Ping its gateway - should succeed) C:\> ping 192.168.20.10 (Ping PC2 in different VLAN - THIS IS THE MAGIC!)

From PC2 (Desktop → Command Prompt):

C:\> ipconfig (Verify IP is 192.168.20.10) C:\> ping 192.168.20.1 (Ping its gateway - should succeed) C:\> ping 192.168.10.10 (Ping PC1 in different VLAN - inter-VLAN routing!)

### Expected Results

- PC1 can ping its gateway (192.168.10.1)

- PC2 can ping its gateway (192.168.20.1)

- PC1 and PC2 can ping each other (inter-VLAN routing working)

### Troubleshooting Tips

- If ping fails within same VLAN → Check port assignment with 'show vlan brief'

- If ping fails between VLANs → Check trunk configuration and router subinterfaces

- If router subinterfaces down → Verify physical interface is 'no shutdown'

## Project 2: DHCP Per VLAN

Duration: 1-2 hours

Objective: Configure the router as a DHCP server to automatically assign IP addresses to devices in each VLAN.

### DHCP Configuration Requirements

Follow these configuration steps in order:

- Exclude IP addresses (FIRST!): ip dhcp excluded-address FIRST_IP LAST_IP

- Create DHCP pool: ip dhcp pool NAME

- Define subnet: network SUBNET SUBNET_MASK

- Define default gateway: default-router IP

- Define DNS server: dns-server IP

Common choice: 8.8.8.8 (Google Public DNS) or 1.1.1.1 (Cloudflare DNS). These public DNS servers translate domain names (like google.com) into IP addresses.

- (Optional) DNS domain name: domain-name NAME

- (Optional) Lease duration: lease DAYS HOURS MINUTES (default: 24 hours)

Important: Always configure excluded addresses FIRST to prevent IP conflicts with static devices (routers, servers, printers).

Understanding Gateway and DNS Server Addresses:

- Default Gateway: Always matches the router’s interface IP address. In VLAN 10, this is 192.168.10.1

- DNS Server: Can be the same as the gateway (if router provides DNS) or different (like 8.8.8.8 for external DNS). Both approaches work!

Example - This Lab (Using External DNS): default-router 192.168.10.1 (router IP) dns-server 8.8.8.8 (Google DNS) Alternative - Router as DNS Server: default-router 192.168.10.1 (router IP) dns-server 192.168.10.1 (same as router - router forwards to 8.8.8.8)

### Router DHCP Configuration

Router> enable Router# configure terminal ! Exclude router gateway addresses from DHCP pools Router(config)# ip dhcp excluded-address 192.168.10.1 Router(config)# ip dhcp excluded-address 192.168.20.1 Router(config)# ip dhcp excluded-address 192.168.99.1 ! DHCP Pool for VLAN 10 (Users) Router(config)# ip dhcp pool VLAN10 Router(dhcp-config)# network 192.168.10.0 255.255.255.0 Router(dhcp-config)# default-router 192.168.10.1 Router(dhcp-config)# dns-server 8.8.8.8 Router(dhcp-config)# exit ! DHCP Pool for VLAN 20 (Servers) Router(config)# ip dhcp pool VLAN20 Router(dhcp-config)# network 192.168.20.0 255.255.255.0 Router(dhcp-config)# default-router 192.168.20.1 Router(dhcp-config)# dns-server 8.8.8.8 Router(dhcp-config)# exit Router(config)# exit Router# write memory

### Verification Steps

- Set PCs to obtain IP automatically (DHCP)

- Verify IP assignment: ipconfig /all (Windows) or ifconfig (Linux)

- Check DHCP bindings on router: show ip dhcp binding

- View pool statistics: show ip dhcp pool

### Common Issues & Solutions

| Issue | Solution |
| --- | --- |
| No IP address received | Check if port is in correct VLAN Verify DHCP pool matches VLAN subnet Check excluded addresses don't block entire range |
| Wrong gateway assigned | Verify default-router statement in DHCP pool Ensure pool name matches intended VLAN |
| IP conflict | Clear DHCP binding: clear ip dhcp binding * Check for static IPs in DHCP range |

## Project 3: NAT/PAT Configuration

Duration: 1-2 hours

Objective: Configure Port Address Translation (PAT) to simulate internet access for internal networks.

Note: If your router has internet access via another interface (e.g., Fa0/1), you can configure real internet access. Otherwise, this simulates the concept.

### NAT/PAT Configuration

Router> enable Router# configure terminal ! Define inside interfaces (VLAN subinterfaces) Router(config)# interface gigabitEthernet 0/0.10 Router(config-subif)# ip nat inside Router(config-subif)# exit Router(config)# interface gigabitEthernet 0/0.20 Router(config-subif)# ip nat inside Router(config-subif)# exit ! Define outside interface (simulated WAN) Router(config)# interface gigabitEthernet 0/1 Router(config-if)# ip address 203.0.113.1 255.255.255.252 Router(config-if)# ip nat outside Router(config-if)# no shutdown Router(config-if)# exit ! Create ACL for NAT Router(config)# access-list 1 permit 192.168.10.0 0.0.0.255 Router(config)# access-list 1 permit 192.168.20.0 0.0.0.255 ! Enable PAT (NAT Overload) Router(config)# ip nat inside source list 1 interface gigabitEthernet 0/1 overload Router(config)# exit Router# write memory

### Verification Commands

- View NAT translations: show ip nat translations

- View NAT statistics: show ip nat statistics

- Clear translations: clear ip nat translation *

### Testing

From a PC, try to ping an address outside your network (or the outside interface IP). Check NAT translations to see the mapping.

## Project 4: Access Control Lists (ACLs)

Duration: 2-3 hours

Objective: Implement network security by controlling traffic flow between VLANs using Access Control Lists.

### Scenario

We want to:

- Block VLAN 10 (Users) from accessing VLAN 20 (Servers) except for specific services

- Allow VLAN 99 (Management) full access to all VLANs

### ACL Configuration

Router> enable Router# configure terminal ! Create Extended ACL to control VLAN 10 access Router(config)# ip access-list extended VLAN10-OUT Router(config-ext-nacl)# remark Block VLAN 10 from VLAN 20 Router(config-ext-nacl)# deny ip 192.168.10.0 0.0.0.255 192.168.20.0 0.0.0.255 Router(config-ext-nacl)# remark Allow all other traffic Router(config-ext-nacl)# permit ip any any Router(config-ext-nacl)# exit ! Apply ACL to VLAN 10 subinterface (outbound direction) Router(config)# interface gigabitEthernet 0/0.10 Router(config-subif)# ip access-group VLAN10-OUT out Router(config-subif)# exit Router(config)# exit Router# write memory

### Advanced Example: Permit Specific Services

! Allow VLAN 10 to access web server (HTTP/HTTPS) in VLAN 20 Router(config)# ip access-list extended VLAN10-OUT Router(config-ext-nacl)# no deny ip 192.168.10.0 0.0.0.255 192.168.20.0 0.0.0.255 Router(config-ext-nacl)# permit tcp 192.168.10.0 0.0.0.255 host 192.168.20.10 eq 80 Router(config-ext-nacl)# permit tcp 192.168.10.0 0.0.0.255 host 192.168.20.10 eq 443 Router(config-ext-nacl)# deny ip 192.168.10.0 0.0.0.255 192.168.20.0 0.0.0.255 Router(config-ext-nacl)# permit ip any any Router(config-ext-nacl)# exit

### Verification & Testing

- View ACL configuration: show ip access-lists

- View ACL application: show ip interface gigabitEthernet 0/0.10

- Test from VLAN 10 PC: Try to ping VLAN 20 PC (should fail)

- Test from VLAN 10 PC: Try to ping router gateway (should succeed)

### Key Concepts

- ACL Direction: 'in' filters incoming traffic, 'out' filters outgoing traffic

- Implicit Deny: ACLs end with an implicit 'deny any' - always add 'permit ip any any' if needed

- Order Matters: ACL processes rules top-to-bottom, first match wins

## Project 5: Switch Security & Hardening

Duration: 2 hours

Objective: Apply enterprise-level security hardening to the switch infrastructure.

### Security Configurations

A. Port Security (Prevent Unauthorized Devices)

Switch> enable Switch# configure terminal ! Configure port security on access port Switch(config)# interface fastEthernet 0/2 Switch(config-if)# switchport mode access Switch(config-if)# switchport port-security Switch(config-if)# switchport port-security maximum 2 Switch(config-if)# switchport port-security violation restrict Switch(config-if)# switchport port-security mac-address sticky Switch(config-if)# exit

B. Disable Unused Ports

! Shutdown all unused interfaces Switch(config)# interface range fastEthernet 0/4 - 24 Switch(config-if-range)# shutdown Switch(config-if-range)# exit

C. Change Native VLAN (Security Best Practice)

! Change native VLAN from default (VLAN 1) to unused VLAN Switch(config)# vlan 999 Switch(config-vlan)# name Unused Switch(config-vlan)# exit Switch(config)# interface gigabitEthernet 0/1 Switch(config-if)# switchport trunk native vlan 999 Switch(config-if)# exit

D. Enable SSH and Disable Telnet

! Set hostname and domain Switch(config)# hostname SW1 SW1(config)# ip domain-name lab.local ! Generate RSA keys for SSH SW1(config)# crypto key generate rsa ! Choose 2048 when prompted ! Enable SSH version 2 SW1(config)# ip ssh version 2 ! Configure VTY lines for SSH only SW1(config)# line vty 0 4 SW1(config-line)# transport input ssh SW1(config-line)# login local SW1(config-line)# exit ! Create local user account SW1(config)# username admin privilege 15 secret Cisco123! SW1(config)# exit

E. Secure Console Access

Switch(config)# line console 0 Switch(config-line)# password ConsolePass123 Switch(config-line)# login Switch(config-line)# logging synchronous Switch(config-line)# exec-timeout 5 0 Switch(config-line)# exit

F. Enable Logging

Switch(config)# logging buffered 16384 Switch(config)# logging console warnings Switch(config)# service timestamps log datetime msec Switch(config)# exit Switch# write memory

### Verification

- Port Security: show port-security interface fa0/2

- SSH Status: show ip ssh

- VTY Lines: show line vty 0 4

- Test SSH: ssh -l admin <switch-IP>

## Project 6: Structured Troubleshooting

Duration: 3-4 hours

Objective: Develop systematic troubleshooting skills by intentionally breaking configurations and restoring them.

### Troubleshooting Methodology

Always follow the OSI model approach:

- Layer 1 (Physical): Check cables, ports, link lights

- Layer 2 (Data Link): Check VLANs, trunks, MAC addresses

- Layer 3 (Network): Check IP addressing, routing, gateways

### Practice Scenarios

Scenario 1: Missing VLAN on Trunk

Break it:

Switch(config)# interface gi0/1 Switch(config-if)# switchport trunk allowed vlan remove 20

Symptoms: VLAN 20 devices can't communicate with other VLANs

Troubleshooting:

- Run: show interfaces trunk

- Check 'VLANs allowed on trunk' - notice VLAN 20 is missing

Fix:

Switch(config)# interface gi0/1 Switch(config-if)# switchport trunk allowed vlan add 20

Scenario 2: Incorrect Default Gateway

Break it:

Configure PC with wrong gateway (e.g., 192.168.10.254 instead of 192.168.10.1)

Symptoms: PC can ping within VLAN but not to other VLANs or router

Troubleshooting:

- Check PC IP configuration: ipconfig /all

- Ping default gateway - fails

- Check router: show ip interface brief - note correct gateway IP

Fix: Correct the gateway on the PC

Scenario 3: NAT Misconfiguration

Break it:

Router(config)# no ip nat inside source list 1 interface fa0/1 overload

Symptoms: Internal hosts have connectivity to router but not to outside network

Troubleshooting:

- Run: show ip nat translations (empty)

- Run: show ip nat statistics (shows 0 translations)

- Check running-config for NAT configuration

Scenario 4: ACL Blocking Legitimate Traffic

Break it:

Router(config)# ip access-list extended VLAN10-OUT Router(config-ext-nacl)# 5 deny ip any any ! This explicit deny blocks everything

Troubleshooting:

- Run: show ip access-lists

- Check hit counts - see which rules are matching

- Remove problematic entry and retest

### Essential Troubleshooting Commands

| Command | Purpose |
| --- | --- |
| show ip interface brief | View all interfaces and IP addresses |
| show vlan brief | Display VLAN assignments |
| show interfaces trunk | Verify trunk configuration |
| show ip route | Display routing table |
| show running-config | View active configuration |
| show ip dhcp binding | View DHCP leases |
| show ip nat translations | View NAT translation table |
| show access-lists | Display ACL configurations |
| ping <IP> | Test Layer 3 connectivity |

## Lab Documentation Guidelines

Your final lab documentation is crucial for interviews and demonstrating your skills. Create a comprehensive document that includes:

### Required Documentation Components

1. Network Diagram

- Create a logical topology showing all devices, VLANs, and connections

- Include IP addresses and interface labels

- Use tools like Draw.io, Visio, or Packet Tracer

2. Configuration Backups

Save complete configurations:

! On Router Router# show running-config ! On Switch Switch# show running-config ! Copy output to text files for documentation

3. IP Addressing Table

Document all IP assignments (already provided in this guide)

4. Troubleshooting Case Studies

Write up at least 2 detailed case studies:

- Problem description

- Symptoms observed

- Troubleshooting steps taken

- Root cause identified

- Resolution and verification

## Next Steps & Career Preparation

### Interview Preparation

Be ready to discuss:

- Why VLANs are used (security, performance, organization)

- Difference between access and trunk ports

- How Router-on-a-Stick works

- NAT vs PAT and when to use each

- ACL placement and direction

- Your troubleshooting methodology

### Resume Enhancement

Add these bullet points to your resume:

- Built multi-VLAN enterprise network lab using Cisco router and switch with Router-on-a-Stick inter-VLAN routing

- Configured DHCP services, NAT/PAT, and extended ACLs using Cisco IOS

- Implemented switch security hardening including port security, SSH access, and unused port shutdown

- Developed structured troubleshooting methodology for Layer 2-3 network connectivity issues

### Expanding Your Skills

After completing these projects, consider:

- Security Focus: Add firewall rules, implement VLAN ACLs, practice penetration testing

- Routing Protocols: Configure OSPF or EIGRP if you add another router

- Advanced Switching: STP, EtherChannel, VTP (if you add another switch)

- Certifications: This lab aligns with CCNA topics - consider certification

## Conclusion

Congratulations on taking the initiative to build real networking skills! By completing these hands-on projects, you're gaining experience that sets you apart from candidates who only have theoretical knowledge.

Remember: The value isn't just in completing the labs—it's in understanding the 'why' behind each configuration, being able to troubleshoot when things break, and confidently explaining your experience in interviews.

Good luck with your networking career!

## Additional Resources

- Cisco IOS Command Reference: https://www.cisco.com/c/en/us/support/ios-nx-os-software/ios-15-4m-t/products-command-reference-list.html

- Packet Tracer (for simulation): https://www.netacad.com/courses/packet-tracer

- CCNA Community: r/ccna on Reddit

- GNS3 (advanced simulation): https://www.gns3.com/