# Network Engineering Home Lab

Getting Started Guide

Cisco Catalyst 2940 Series Switch & Cisco 800 Series Router

## Introduction

This guide will help you set up and complete a professional-level home networking lab using physical Cisco equipment. By the end of this series, you'll have hands-on experience with VLANs, routing, DHCP, NAT/PAT, ACLs, and security hardening—all skills directly applicable to network and security engineering roles.

Each project builds on the previous one, creating a comprehensive understanding of enterprise network configuration and troubleshooting.

## Lab Equipment

- Cisco 800 Series Router (IOS-based)

- Cisco Catalyst 2940 Series Switch (IOS-based)

- Ethernet cables (CAT5e or better)

- Console cable (RS-232 or USB-to-Serial adapter)

- 1-2 PCs/laptops for testing (can use VMs)

## Pre-Lab Setup & Preparation

### Step 1: Physical Connections

- Router Console Connection: Connect your PC to the router's console port using a console cable

- Switch Console Connection: Connect to the switch console port

- Network Connections: Connect router interface (e.g., FastEthernet0/0) to switch (e.g., GigabitEthernet0/1) using an Ethernet cable

### Step 2: Terminal Emulator Setup

You'll need terminal software to connect to your devices:

- Windows: PuTTY or TeraTerm

- macOS/Linux: Screen or Minicom (built-in)

Connection Settings: Baud rate 9600, Data bits 8, Stop bits 1, Parity None, Flow control None

### Step 3: Initial Device Access

Test connectivity to both devices:

- Open terminal emulator with settings above

- Power on the router—you should see boot messages

- Press Enter to see the router prompt (Router> or Router#)

- Repeat for the switch

## Essential IOS Commands to Know

Before starting projects, familiarize yourself with these Cisco IOS modes and commands:

| Mode | Description & Key Commands |
| --- | --- |
| User EXEC Router> | Limited view mode. Type 'enable' to enter privileged mode. |
| Privileged EXEC Router# | Full viewing rights. Key commands: show running-config, show ip interface brief, show vlan brief |
| Global Config Router(config)# | Configuration mode. Enter with 'configure terminal'. Exit with 'exit' or Ctrl+Z. |
| Interface Config Router(config-if)# | Configure specific interface. Enter with 'interface <type> <number>'. |

Tip: Use TAB for command completion and '?' for help at any time.

## Lab Network Design

Your lab will implement this logical network design:

| VLAN | Name | Purpose |
| --- | --- | --- |
| 10 | Users | End-user workstations |
| 20 | Servers | Server infrastructure |
| 99 | Management | Network device management |

| VLAN | Network | Gateway | DHCP Range |
| --- | --- | --- | --- |
| 10 | 192.168.10.0/24 | 192.168.10.1 | .10-.250 |
| 20 | 192.168.20.0/24 | 192.168.20.1 | .10-.250 |
| 99 | 192.168.99.0/24 | 192.168.99.1 | Static only |

Physical Topology:

- PC1 → Switch Port Fa0/2 (VLAN 10)

- PC2 → Switch Port Fa0/3 (VLAN 20)

- Switch Gi0/1 → Router Fa0/0 (802.1Q Trunk)

## Project 1: VLANs, Trunking, and Inter-VLAN Routing

Duration: 2-3 hours

Objective: Create network segmentation using VLANs and enable communication between VLANs using Router-on-a-Stick.

### Configuration Tasks

A. Switch Configuration

Create VLANs:

Switch> enable Switch# configure terminal Switch(config)# vlan 10 Switch(config-vlan)# name Users Switch(config-vlan)# exit Switch(config)# vlan 20 Switch(config-vlan)# name Servers Switch(config-vlan)# exit Switch(config)# vlan 99 Switch(config-vlan)# name Management Switch(config-vlan)# exit

Assign Access Ports:

Switch(config)# interface fastEthernet 0/2 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 10 Switch(config-if)# exit Switch(config)# interface fastEthernet 0/3 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 20 Switch(config-if)# exit

Configure Trunk Port:

Switch(config)# interface gigabitEthernet 0/1 Switch(config-if)# switchport mode trunk Switch(config-if)# switchport trunk encapsulation dot1q Switch(config-if)# switchport trunk allowed vlan 10,20,99 Switch(config-if)# exit Switch(config)# exit Switch# write memory

B. Router Configuration (Router-on-a-Stick)

Router> enable Router# configure terminal Router(config)# interface fastEthernet 0/0 Router(config-if)# no shutdown Router(config-if)# exit Router(config)# interface fastEthernet 0/0.10 Router(config-subif)# encapsulation dot1Q 10 Router(config-subif)# ip address 192.168.10.1 255.255.255.0 Router(config-subif)# exit Router(config)# interface fastEthernet 0/0.20 Router(config-subif)# encapsulation dot1Q 20 Router(config-subif)# ip address 192.168.20.1 255.255.255.0 Router(config-subif)# exit Router(config)# interface fastEthernet 0/0.99 Router(config-subif)# encapsulation dot1Q 99 Router(config-subif)# ip address 192.168.99.1 255.255.255.0 Router(config-subif)# exit Router(config)# exit Router# write memory

C. Configure Switch Management IP (VLAN 99)

Configure the switch with a management IP address so you can access it remotely via SSH or Telnet from any VLAN.

Switch> enable Switch# configure terminal Switch(config)# interface vlan 99 Switch(config-if)# ip address 192.168.99.2 255.255.255.0 Switch(config-if)# description Management Interface Switch(config-if)# no shutdown Switch(config-if)# exit Switch(config)# ip default-gateway 192.168.99.1 Switch(config)# exit Switch# write memory

Management IP Details:

- Switch IP: 192.168.99.2

- Gateway: 192.168.99.1 (the router)

- Purpose: Allows remote management of the switch via SSH/Telnet from any VLAN

Note: SSH and Telnet configuration will come in Project 5 (Switch Security). For now, you have console access and the management IP is configured.

D. Configure Test Devices with Static IP Addresses

Configure your test PCs/laptops with static IP addresses. Remember, we haven’t set up DHCP yet, so you must manually configure each device.

Device 1 Configuration (VLAN 10 - Users):

Connect to switch port assigned to VLAN 10, then configure:

IP Address: 192.168.10.10 Subnet Mask: 255.255.255.0 Default Gateway: 192.168.10.1

Device 2 Configuration (VLAN 20 - Servers):

Connect to switch port assigned to VLAN 20, then configure:

IP Address: 192.168.20.10 Subnet Mask: 255.255.255.0 Default Gateway: 192.168.20.1

Note: Configure static IPs on your test devices (Windows: Network Adapter Settings, macOS: System Preferences → Network, Linux: nmtui or /etc/network/interfaces). DHCP configuration comes in Project 2.

### E. Verification Steps

- Verify VLANs: show vlan brief

- Verify Trunk: show interfaces trunk

- Verify Subinterfaces: show ip interface brief

- Test Connectivity: Configure PC1 with IP 192.168.10.10/24, gateway 192.168.10.1

- Configure PC2 with IP 192.168.20.10/24, gateway 192.168.20.1, then ping between them

### Expected Results

- PC1 can ping its gateway (192.168.10.1)

- PC2 can ping its gateway (192.168.20.1)

- PC1 and PC2 can ping each other (inter-VLAN routing working)

### Troubleshooting Tips

- If ping fails within same VLAN → Check port assignment with 'show vlan brief'

- If ping fails between VLANs → Check trunk configuration and router subinterfaces

- If router subinterfaces down → Verify physical interface is 'no shutdown'

## Project 2: DHCP Per VLAN

Duration: 1-2 hours

Objective: Configure the router as a DHCP server to automatically assign IP addresses to devices in each VLAN.

### DHCP Configuration Requirements

When configuring DHCP on a Cisco router, follow these steps in order:

- FIRST - Exclude IP addresses using: ip dhcp excluded-address FIRST_IP LAST_IP

- Create a DHCP pool: ip dhcp pool NAME

- Define the subnet: network SUBNET SUBNET_MASK

- Define the default gateway: default-router IP

- Define the DNS server: dns-server IP

Common choice: 8.8.8.8 (Google Public DNS) or 1.1.1.1 (Cloudflare DNS). These public DNS servers translate domain names (like google.com) into IP addresses.

- (Optional) DNS domain name: domain-name NAME

- (Optional) Lease duration: lease DAYS HOURS MINUTES (Default: 24 hours)

Important: Always configure excluded addresses FIRST before creating DHCP pools to prevent conflicts with static IPs.

Understanding Gateway and DNS Server Addresses:

- Default Gateway: Always matches the router’s interface IP address. In VLAN 10, this is 192.168.10.1

- DNS Server: Can be the same as the gateway (if router provides DNS) or different (like 8.8.8.8 for external DNS). Both approaches work!

Example - This Lab (Using External DNS): default-router 192.168.10.1 (router IP) dns-server 8.8.8.8 (Google DNS) Alternative - Router as DNS Server: default-router 192.168.10.1 (router IP) dns-server 192.168.10.1 (same as router - router forwards to 8.8.8.8)

### Router DHCP Configuration

Router> enable Router# configure terminal ! Exclude router gateway addresses from DHCP pools Router(config)# ip dhcp excluded-address 192.168.10.1 Router(config)# ip dhcp excluded-address 192.168.20.1 Router(config)# ip dhcp excluded-address 192.168.99.1 ! DHCP Pool for VLAN 10 (Users) Router(config)# ip dhcp pool VLAN10 Router(dhcp-config)# network 192.168.10.0 255.255.255.0 Router(dhcp-config)# default-router 192.168.10.1 Router(dhcp-config)# dns-server 8.8.8.8 Router(dhcp-config)# exit ! DHCP Pool for VLAN 20 (Servers) Router(config)# ip dhcp pool VLAN20 Router(dhcp-config)# network 192.168.20.0 255.255.255.0 Router(dhcp-config)# default-router 192.168.20.1 Router(dhcp-config)# dns-server 8.8.8.8 Router(dhcp-config)# exit Router(config)# exit Router# write memory

### Verification Steps

- Set PCs to obtain IP automatically (DHCP)

- Verify IP assignment: ipconfig /all (Windows) or ifconfig (Linux)

- Check DHCP bindings on router: show ip dhcp binding

- View pool statistics: show ip dhcp pool

### Common Issues & Solutions

| Issue | Solution |
| --- | --- |
| No IP address received | Check if port is in correct VLAN Verify DHCP pool matches VLAN subnet Check excluded addresses don't block entire range |
| Wrong gateway assigned | Verify default-router statement in DHCP pool Ensure pool name matches intended VLAN |
| IP conflict | Clear DHCP binding: clear ip dhcp binding * Check for static IPs in DHCP range |

## Project 3: NAT/PAT Configuration

Duration: 1-2 hours

Objective: Configure Port Address Translation (PAT) to simulate internet access for internal networks.

Note: If your router has internet access via another interface (e.g., Fa0/1), you can configure real internet access. Otherwise, this simulates the concept.

### NAT/PAT Configuration

Router> enable Router# configure terminal ! Define inside interfaces (VLAN subinterfaces) Router(config)# interface fastEthernet 0/0.10 Router(config-subif)# ip nat inside Router(config-subif)# exit Router(config)# interface fastEthernet 0/0.20 Router(config-subif)# ip nat inside Router(config-subif)# exit ! Define outside interface (simulated WAN) Router(config)# interface fastEthernet 0/1 Router(config-if)# ip address 203.0.113.1 255.255.255.252 Router(config-if)# ip nat outside Router(config-if)# no shutdown Router(config-if)# exit ! Create ACL for NAT Router(config)# access-list 1 permit 192.168.10.0 0.0.0.255 Router(config)# access-list 1 permit 192.168.20.0 0.0.0.255 ! Enable PAT (NAT Overload) Router(config)# ip nat inside source list 1 interface fastEthernet 0/1 overload Router(config)# exit Router# write memory

### Verification Commands

- View NAT translations: show ip nat translations

- View NAT statistics: show ip nat statistics

- Clear translations: clear ip nat translation *

### Testing

From a PC, try to ping an address outside your network (or the outside interface IP). Check NAT translations to see the mapping.

## Project 4: Access Control Lists (ACLs)

Duration: 2-3 hours

Objective: Implement network security by controlling traffic flow between VLANs using Access Control Lists.

### Scenario

We want to:

- Block VLAN 10 (Users) from accessing VLAN 20 (Servers) except for specific services

- Allow VLAN 99 (Management) full access to all VLANs

### ACL Configuration

Router> enable Router# configure terminal ! Create Extended ACL to control VLAN 10 access Router(config)# ip access-list extended VLAN10-OUT Router(config-ext-nacl)# remark Block VLAN 10 from VLAN 20 Router(config-ext-nacl)# deny ip 192.168.10.0 0.0.0.255 192.168.20.0 0.0.0.255 Router(config-ext-nacl)# remark Allow all other traffic Router(config-ext-nacl)# permit ip any any Router(config-ext-nacl)# exit ! Apply ACL to VLAN 10 subinterface (outbound direction) Router(config)# interface fastEthernet 0/0.10 Router(config-subif)# ip access-group VLAN10-OUT out Router(config-subif)# exit Router(config)# exit Router# write memory

### Advanced Example: Permit Specific Services

! Allow VLAN 10 to access web server (HTTP/HTTPS) in VLAN 20 Router(config)# ip access-list extended VLAN10-OUT Router(config-ext-nacl)# no deny ip 192.168.10.0 0.0.0.255 192.168.20.0 0.0.0.255 Router(config-ext-nacl)# permit tcp 192.168.10.0 0.0.0.255 host 192.168.20.10 eq 80 Router(config-ext-nacl)# permit tcp 192.168.10.0 0.0.0.255 host 192.168.20.10 eq 443 Router(config-ext-nacl)# deny ip 192.168.10.0 0.0.0.255 192.168.20.0 0.0.0.255 Router(config-ext-nacl)# permit ip any any Router(config-ext-nacl)# exit

### Verification & Testing

- View ACL configuration: show ip access-lists

- View ACL application: show ip interface fastEthernet 0/0.10

- Test from VLAN 10 PC: Try to ping VLAN 20 PC (should fail)

- Test from VLAN 10 PC: Try to ping router gateway (should succeed)

### Key Concepts

- ACL Direction: 'in' filters incoming traffic, 'out' filters outgoing traffic

- Implicit Deny: ACLs end with an implicit 'deny any' - always add 'permit ip any any' if needed

- Order Matters: ACL processes rules top-to-bottom, first match wins

## Project 5: Switch Security & Hardening

Duration: 2 hours

Objective: Apply enterprise-level security hardening to the switch infrastructure.

### Security Configurations

A. Port Security (Prevent Unauthorized Devices)

Switch> enable Switch# configure terminal ! Configure port security on access port Switch(config)# interface fastEthernet 0/2 Switch(config-if)# switchport mode access Switch(config-if)# switchport port-security Switch(config-if)# switchport port-security maximum 2 Switch(config-if)# switchport port-security violation restrict Switch(config-if)# switchport port-security mac-address sticky Switch(config-if)# exit

B. Disable Unused Ports

! Shutdown all unused interfaces Switch(config)# interface range fastEthernet 0/4 - 24 Switch(config-if-range)# shutdown Switch(config-if-range)# exit

C. Change Native VLAN (Security Best Practice)

! Change native VLAN from default (VLAN 1) to unused VLAN Switch(config)# vlan 999 Switch(config-vlan)# name Unused Switch(config-vlan)# exit Switch(config)# interface gigabitEthernet 0/1 Switch(config-if)# switchport trunk native vlan 999 Switch(config-if)# exit

D. Enable SSH and Disable Telnet

! Set hostname and domain Switch(config)# hostname SW1 SW1(config)# ip domain-name lab.local ! Generate RSA keys for SSH SW1(config)# crypto key generate rsa ! Choose 2048 when prompted ! Enable SSH version 2 SW1(config)# ip ssh version 2 ! Configure VTY lines for SSH only SW1(config)# line vty 0 4 SW1(config-line)# transport input ssh SW1(config-line)# login local SW1(config-line)# exit ! Create local user account SW1(config)# username admin privilege 15 secret Cisco123! SW1(config)# exit

E. Secure Console Access

Switch(config)# line console 0 Switch(config-line)# password ConsolePass123 Switch(config-line)# login Switch(config-line)# logging synchronous Switch(config-line)# exec-timeout 5 0 Switch(config-line)# exit

F. Enable Logging

Switch(config)# logging buffered 16384 Switch(config)# logging console warnings Switch(config)# service timestamps log datetime msec Switch(config)# exit Switch# write memory

### Verification

- Port Security: show port-security interface fa0/2

- SSH Status: show ip ssh

- VTY Lines: show line vty 0 4

- Test SSH: ssh -l admin <switch-IP>

## Project 6: Structured Troubleshooting

Duration: 3-4 hours

Objective: Develop systematic troubleshooting skills by intentionally breaking configurations and restoring them.

### Troubleshooting Methodology

Always follow the OSI model approach:

- Layer 1 (Physical): Check cables, ports, link lights

- Layer 2 (Data Link): Check VLANs, trunks, MAC addresses

- Layer 3 (Network): Check IP addressing, routing, gateways

### Practice Scenarios

Scenario 1: Missing VLAN on Trunk

Break it:

Switch(config)# interface gi0/1 Switch(config-if)# switchport trunk allowed vlan remove 20

Symptoms: VLAN 20 devices can't communicate with other VLANs

Troubleshooting:

- Run: show interfaces trunk

- Check 'VLANs allowed on trunk' - notice VLAN 20 is missing

Fix:

Switch(config)# interface gi0/1 Switch(config-if)# switchport trunk allowed vlan add 20

Scenario 2: Incorrect Default Gateway

Break it:

Configure PC with wrong gateway (e.g., 192.168.10.254 instead of 192.168.10.1)

Symptoms: PC can ping within VLAN but not to other VLANs or router

Troubleshooting:

- Check PC IP configuration: ipconfig /all

- Ping default gateway - fails

- Check router: show ip interface brief - note correct gateway IP

Fix: Correct the gateway on the PC

Scenario 3: NAT Misconfiguration

Break it:

Router(config)# no ip nat inside source list 1 interface fa0/1 overload

Symptoms: Internal hosts have connectivity to router but not to outside network

Troubleshooting:

- Run: show ip nat translations (empty)

- Run: show ip nat statistics (shows 0 translations)

- Check running-config for NAT configuration

Scenario 4: ACL Blocking Legitimate Traffic

Break it:

Router(config)# ip access-list extended VLAN10-OUT Router(config-ext-nacl)# 5 deny ip any any ! This explicit deny blocks everything

Troubleshooting:

- Run: show ip access-lists

- Check hit counts - see which rules are matching

- Remove problematic entry and retest

### Essential Troubleshooting Commands

| Command | Purpose |
| --- | --- |
| show ip interface brief | View all interfaces and IP addresses |
| show vlan brief | Display VLAN assignments |
| show interfaces trunk | Verify trunk configuration |
| show ip route | Display routing table |
| show running-config | View active configuration |
| show ip dhcp binding | View DHCP leases |
| show ip nat translations | View NAT translation table |
| show access-lists | Display ACL configurations |
| ping <IP> | Test Layer 3 connectivity |

## Lab Documentation Guidelines

Your final lab documentation is crucial for interviews and demonstrating your skills. Create a comprehensive document that includes:

### Required Documentation Components

1. Network Diagram

- Create a logical topology showing all devices, VLANs, and connections

- Include IP addresses and interface labels

- Use tools like Draw.io, Visio, or Packet Tracer

2. Configuration Backups

Save complete configurations:

! On Router Router# show running-config ! On Switch Switch# show running-config ! Copy output to text files for documentation

3. IP Addressing Table

Document all IP assignments (already provided in this guide)

4. Troubleshooting Case Studies

Write up at least 2 detailed case studies:

- Problem description

- Symptoms observed

- Troubleshooting steps taken

- Root cause identified

- Resolution and verification

## Next Steps & Career Preparation

### Interview Preparation

Be ready to discuss:

- Why VLANs are used (security, performance, organization)

- Difference between access and trunk ports

- How Router-on-a-Stick works

- NAT vs PAT and when to use each

- ACL placement and direction

- Your troubleshooting methodology

### Resume Enhancement

Add these bullet points to your resume:

- Built multi-VLAN enterprise network lab using Cisco router and switch with Router-on-a-Stick inter-VLAN routing

- Configured DHCP services, NAT/PAT, and extended ACLs using Cisco IOS

- Implemented switch security hardening including port security, SSH access, and unused port shutdown

- Developed structured troubleshooting methodology for Layer 2-3 network connectivity issues

### Expanding Your Skills

After completing these projects, consider:

- Security Focus: Add firewall rules, implement VLAN ACLs, practice penetration testing

- Routing Protocols: Configure OSPF or EIGRP if you add another router

- Advanced Switching: STP, EtherChannel, VTP (if you add another switch)

- Certifications: This lab aligns with CCNA topics - consider certification

## Conclusion

Congratulations on taking the initiative to build real networking skills! By completing these hands-on projects, you're gaining experience that sets you apart from candidates who only have theoretical knowledge.

Remember: The value isn't just in completing the labs—it's in understanding the 'why' behind each configuration, being able to troubleshoot when things break, and confidently explaining your experience in interviews.

Good luck with your networking career!

## Additional Resources

- Cisco IOS Command Reference: https://www.cisco.com/c/en/us/support/ios-nx-os-software/ios-15-4m-t/products-command-reference-list.html

- Packet Tracer (for simulation): https://www.netacad.com/courses/packet-tracer

- CCNA Community: r/ccna on Reddit

- GNS3 (advanced simulation): https://www.gns3.com/